CPEN 291 
Mini-Project Report

A. Group info

Lab section: L2A  (L2A or L2B)	                    				Group #: A-G4   (Example: A-G1)	
Group’s Lab Bench #s: 9 and 4    (Example: 1 and 2)          
Student names:

| Andrea Shao | Brielle Law |
| --- | --- |
| Matthew Chow | Matthew Stefansson |
| Matthew Yen | Sebastian Gonzalez |

All the underlined text in this template are for your info. You should remove any text that is underlined before submission to Canvas.

Please keep the format of the report as is (e.g. do not omit any section, or change the font size or margins).

In Sections B and C of the report:

- Explain the design and implementation procedures, and thoroughly provide documentation for the circuitry and software. During the project, you must have selected any method over another for some steps, describe the alternative (e.g. the second best) you considered.  Include block diagrams or drawing to identify the main components and their interactions.

- You may include code segments in this part of the report only whenever needed for the explanations of the software design and approach. Your code must include comment statements, so do not repeat what is already included in the comment statements. As usual you will need to submit the complete code file separately, and also to include the complete code as an appendix to this report. The code must be readable in the first place and include sufficient comments (per code segment and per line, when needed) for documentation.

B. Technical documentation for the main functionality

Fully document your design and implementation for the main functionality here. In particular explain:

- What the six dance moves are, and how they are different

  - Dance 1: The robot taps its left foot a few times, and then shakes its right foot. It then taps its right foot a few times and then shakes its left foot. This movement is then repeated, rotating between the two feet.

  - Dance 2: The robot points its feet outwards and then inwards in a quick sweep before beginning to tap and alternate between its two feet. It taps for a total of 6 times before repeating the sweep and tapping cycle.

  - Dance 3: The robot initially has its feet facing outward and slightly upward. It then quickly turns its feet in towards the center of its body and changes the angle of its feet. By doing this, it jumps momentarily off the ground. It then quickly shifts its feet back to the starting position.

  - Dance 4: The robot raises its left and right foot in sync. The feet will tilt right and left, slightly raising the robot. Next, the legs will rotate in half circles, moving the feet outwards then inwards. The robot repeats this movement at different speeds. After this, it will return to the starting position.

  - Dance 5: The two feet both shift right then left and repeats this shifting movement. As it shifts its feet, it also moves backwards at the same time. Overall, this dance move appears like a zig zag backwards shuffle.

  - Dance 6:  The robot shifts the bottom motors to make the robot look like its standing on its tippy toes. To do this, it rotates its feet inwards changes the angle of the feet dramatically so its raised high off the group. It then returns to its original position and repeats this movement.

  - Ending pose: The robot’s dances end in the splits position.

- How you have implemented each of the six robot moves

To implement the dance moves, each move was developed in different files (dance1, dance 2 etc). All the separate files were thoroughly tested and adjusted as necessary. Once all the subfiles were completed, they were integrated into the main coding file of the robot dancing. For a smooth process and to make the robot “dance” more naturally, we added the threading with the piezo buzzer and ultrasonic sensor.

- What info the LCD displays and how it has been mounted/attached for optimal viewing

The LCD display will show and add certain elements to our dancing robot. The LCD is mounted on the breadboard (which sits inside the robot’s shell) so that it can easily be viewed. When the robot is first turned on, an opening image of a brain and the words “Hi, I’m bub” are shown. As the robot shuffles through its different dance moves, the appropriate dance name appears, as well as the correct pose. The introduction photo and a few sample dance photos can be seen below.

C. Technical documentation for the additional functionality

Fully document your design and implementation for the additional functionality here. Explain:

- What the additional functionalities are

  - We added the piezo buzzer to play a song while our robot is dancing

  - We added the HC-SR04 ultrasonic sensor to play a song when it is less than 4cm from an object and stop playing the song when it is more than 25cm from an object.

- Include the list of the additional components you used

  - 1 Piezo Buzzer

  - 1 HC-SR04 ultrasonic sensor

- The hardware implementation

  - The positive end of the buzzer is plug into port A0

  - The trigger is plug into D3 and the echo is plugged into D4

  - Both the buzzer and ultrasonic sensor are plugged into ground and the ultrasonic sensor is plugged into the power

- The software implementation

  - In order to implement the software of each dance move, it would seem like there were two thread running, however, there was only one. To be checking the outer environment constantly, we would switch between moving the robot, checking the ultrasonic sensor and playing the buzzer. To play the buzzer, we have two lists; one for the note and another one for how long the note plays. Every time a note is played, the index was incremented. At the beginning of each dance move, the song is checked to determine if the song has finished and the song restarts (where the index goes to zero). For the ultrasonic sensor, we would check if the distance was less than 4cm to determine if it plays a song. While using the buzzer and ultrasonic sensor, the robot would continue to dance. Once one cycle was done, the software would re-check the components, and continue to do this in a loop.

D. Test and evaluations

Explain your evaluation and testing procedures for hardware and software. Please demonstrate systematic testing, debugging and continuous integration. Include the problems you have encountered and how you resolve them, as well as best practices you have incorporated.

While doing this project, our goal was to have a bug free robot. This demonstrates the need for testing and shows its importance to our overall product. To ensure that our integration easily allows for debugging, every individual component was thoroughly tested before it was added into our main functionality. For example, when preparing the robot, we tested each servo motor with the previously written raspberry pi code before implementing putting the new servo onto the robot. By doing this, we found out that a motor was not working and was able to replace it before testing our newly developed code.

With respect to testing with the code, each partition was coded separately and tested rigorously. This made it simple and easy to find errors in the code and allowed for solutions to be implemented. From there, we would integrate that part of the code into the main robot function. This process reduced the number of bugs in our code and allowed us to speed up the integration stages. When we did find bugs, we were able to remove it from integration and test it separately again before reintroducing it into our project.

When analyzing initial concerns with the robot, the first one that popped to mind was how the motors were receiving power. Only three motors can run at a time. At first, we attempted to reduce the noise by adding capacitors, but this failed to work. To fix this problem, we looked more closely at the documentation of the motors and the ItsyBitsy. Once doing this, we realized that you could provide power using the USB power source, not the 3V power source, and this resolved all of our concerns.

E. Conclusions and Reflections

Reflect and conclude on the lessons, tricks or interesting concepts you have learned during the project.

Also reflect on other aspects such as team work, project management, time management, ...

As this was our first project working as a full group, we started off by finding our respective roles that made us most comfortable and testing out the new group dynamic. Roles of leaders, mediators, and others came quite naturally to us and we were very quickly able to act as a fully functioning team. We easily divided different tasks amongst ourselves and split off during lab hours to work on our tasks. The management did not fall on just one person’s shoulders, but rather the task delegation was taken up by almost everyone. As for time management, we had a bit of trouble with this in the beginning as we all have the same upcoming deadlines and exam schedule. But we were able to still find days between exams to meet after class to work on our project.

As for what we learned regarding the project itself, this was the first time for some of our group members to be able to work on code that produced tangible movement. As a lot of experience we’ve had up until now has purely been displayed on computer screens, this was a great introduction project to see the connections between software and physical outcomes. We learned how we could produce intricate and exciting dance moves by building upon our simple functionalities. By simply just adjusting angles on motors, we were able to create patterns and repetitions in movement and create something both entertaining and fluid to watch. Overall, the open-endedness of the project also allowed for a lot of creative designing and processes. We were able to experiment with buzzers and keypads and choose which features we wanted to include in our end product. We’ve learned about many different designing strategies throughout first and second year and, from this project, we were able to put this knowledge to use and actually experience working with an iterative design process.

F. References and bibliography

Provide any relevant references.

Also include the list and description of the files submitted for this lab (including code and Fritzing breadboard schematics)

On our github, as individual members attempted writing dance moves themselves, our dances are all in separate files. We also created the buzzer and sonar stopping program separately. In submission, we have combined all of the dance moves and additional features into one main program.

- Main with dance moves, buzzer, and sonar

- Fritzing (breadboard view)

- Lab report (this file)

Include the following appendixes. There is no page limit for the appendixes.

Appendix A – Robot pictures

Include a few pictures of your robot here. The pictures should clearly show the robot as a whole, as well as all electronics, wiring and parts.  Include photos taken from the top, and from the sides. Show the location/installation of circuits and components as clearly as possible.

Appendix B – Code

All the code is clearly commented. In github, you can see that all the different parts of the robot were initially tested in their own files. This is was done whether it was a dance, buzzer or the integration between multiple components.

Include the complete CircuitPython code with comment statements. This code must be the same code as the files you demo and submit. Clearly identify the portion of the code for the main functionality and the Additional functionality.

The code must be readable, with indentation, syntax highlighting (that is, copy with colour coding), and on white background. The code must be in text (that is, absolutely no snapshots of the code).

# adding import statements

import board

import displayio

import terminalio

from adafruit_display_text import label

from adafruit_st7735r import ST7735R

import adafruit_imageload

import time

import pulseio

from adafruit_motor import servo

import simpleio

import adafruit_hcsr04

# create a PWMOut object on Pin A2.

# can change inputs board.A1 etc.

pwm1 = pulseio.PWMOut(board.D13, duty_cycle=2 ** 15, frequency=50)

pwm2 = pulseio.PWMOut(board.D12, duty_cycle=2 ** 15, frequency=50)

pwm3 = pulseio.PWMOut(board.D10, duty_cycle=2 ** 15, frequency=50)

pwm4 = pulseio.PWMOut(board.D5, duty_cycle=2 ** 15, frequency=50)

# Create a servo object, my_servo.

lowLeft = servo.Servo(pwm1) #alternative name servo1

lowRight = servo.Servo(pwm2) #alternative name servo2

upRight = servo.Servo(pwm3) #alternative name servo3

upLeft = servo.Servo(pwm4) #alternative name servo4

# setting up sonar to use pin out D3 and D4

sonar = adafruit_hcsr04.HCSR04(trigger_pin=board.D3, echo_pin=board.D4)

# Release any resources currently in use for the displays

displayio.release_displays()

# setting up LCD board

spi = board.SPI()

tft_cs = board.D11

tft_dc = board.D9

# setting up the LCD board so the code knows the size of the screen

display_bus = displayio.FourWire(spi, command=tft_dc, chip_select=tft_cs, reset=board.D7)

display = ST7735R(display_bus, width=128, height=128)

# Define pin connected to piezo buzzer.

# creating a bunch of note varriables to help create the song

PIEZO_PIN = board.A1

NOTE_B0  =31

NOTE_C1  =33

NOTE_CS1 =35

NOTE_D1  =37

NOTE_DS1 =39

NOTE_E1  =41

NOTE_F1  =44

NOTE_FS1 =46

NOTE_G1  =49

NOTE_GS1 =52

NOTE_A1  =55

NOTE_AS1 =58

NOTE_B1  =62

NOTE_C2  =65

NOTE_CS2 =69

NOTE_D2  =73

NOTE_DS2 =78

NOTE_E2  =82

NOTE_F2  =87

NOTE_FS2 =93

NOTE_G2  =98

NOTE_GS2 =104

NOTE_A2  =110

NOTE_AS2 =117

NOTE_B2  =123

NOTE_C3  =131

NOTE_CS3 =139

NOTE_D3  =147

NOTE_DS3 =156

NOTE_E3  =165

NOTE_F3  =175

NOTE_FS3 =185

NOTE_G3  =196

NOTE_GS3 =208

NOTE_A3  =220

NOTE_AS3 =233

NOTE_B3  =247

NOTE_C4  =262

NOTE_CS4 =277

NOTE_D4  =294

NOTE_DS4 =311

NOTE_E4  =330

NOTE_F4  =349

NOTE_FS4 =370

NOTE_G4  =392

NOTE_GS4 =415

NOTE_A4  =440

NOTE_AS4 =466

NOTE_B4  =494

NOTE_C5  =523

NOTE_CS5 =554

NOTE_D5  =587

NOTE_DS5 =622

NOTE_E5  =659

NOTE_F5  =698

NOTE_FS5 =740

NOTE_G5  =784

NOTE_GS5 =831

NOTE_A5  =880

NOTE_AS5 =932

NOTE_B5  =988

NOTE_C6  =1047

NOTE_CS6 =1109

NOTE_D6  =1175

NOTE_DS6 =1245

NOTE_E6  =1319

NOTE_F6  =1397

NOTE_FS6 =1480

NOTE_G6  =1568

NOTE_GS6 =1661

NOTE_A6  =1760

NOTE_AS6 =1865

NOTE_B6  =1976

NOTE_C7  =2093

NOTE_CS7 =2217

NOTE_D7  =2349

NOTE_DS7 =2489

NOTE_E7  =2637

NOTE_F7  =2794

NOTE_FS7 =2960

NOTE_G7  =3136

NOTE_GS7 =3322

NOTE_A7  =3520

NOTE_AS7 =3729

NOTE_B7  =3951

NOTE_C8  =4186

NOTE_CS8 =4435

NOTE_D8  =4699

NOTE_DS8 =4978

# this allows us to set up how long each note plays for

wholeNote = 2

halfNote = 1

quarterNote = 0.5

eighthNote = 0.25

sixteenthNote = 0.125

# creating a list of notes

notes = [NOTE_C4, NOTE_C4, NOTE_D4, NOTE_C4, NOTE_F4, NOTE_E4]

# this is a list for our song shootingStars this list of notes corresponds with the list of notes in the shooting Stars Beats

shootingStarsNotes = [

NOTE_DS5, NOTE_DS5, 0, NOTE_E5, NOTE_B4, NOTE_GS4, NOTE_DS5,

NOTE_DS5, NOTE_DS5, 0, NOTE_E5, NOTE_B4, NOTE_GS4, NOTE_DS5,

NOTE_DS5, NOTE_DS5, 0, NOTE_E5, NOTE_B4, NOTE_GS4, NOTE_DS5,

NOTE_DS5, NOTE_DS5, 0, NOTE_E5, NOTE_DS5, NOTE_B4, NOTE_GS4,

NOTE_FS5, NOTE_FS5, NOTE_GS5, NOTE_DS5, NOTE_B4, NOTE_FS5,

NOTE_FS5, NOTE_FS5, NOTE_GS5, NOTE_DS5, NOTE_B4, NOTE_FS5,

NOTE_FS5, NOTE_FS5, NOTE_GS5, NOTE_DS5, NOTE_B4, NOTE_FS5,

NOTE_FS5, NOTE_FS5, NOTE_DS5, NOTE_E5, NOTE_DS5, NOTE_B4, NOTE_GS4,

NOTE_DS5, NOTE_DS5, 0, NOTE_E5, NOTE_B4, NOTE_GS4, NOTE_DS5,

NOTE_DS5, NOTE_DS5, 0, NOTE_E5, NOTE_B4, NOTE_GS4, NOTE_DS5,

NOTE_DS5, NOTE_DS5, 0, NOTE_E5, NOTE_B4, NOTE_GS4, NOTE_DS5,

NOTE_DS5, NOTE_DS5, 0, NOTE_E5, NOTE_DS5, NOTE_B4, NOTE_GS4

]

# this is a list that has the length of each note above

shootingStarsBeats = [

quarterNote+eighthNote, sixteenthNote, sixteenthNote, quarterNote, eighthNote, sixteenthNote, sixteenthNote,

quarterNote+eighthNote, sixteenthNote, sixteenthNote, quarterNote, eighthNote, sixteenthNote, sixteenthNote,

quarterNote+eighthNote, sixteenthNote, sixteenthNote, quarterNote, eighthNote, sixteenthNote, sixteenthNote,

quarterNote+eighthNote, sixteenthNote, sixteenthNote, eighthNote, eighthNote, eighthNote, eighthNote,

quarterNote+eighthNote, eighthNote, quarterNote, eighthNote, sixteenthNote, sixteenthNote,

quarterNote+eighthNote, eighthNote, quarterNote, eighthNote, sixteenthNote, sixteenthNote,

quarterNote+eighthNote, eighthNote, quarterNote, eighthNote, sixteenthNote, sixteenthNote,

quarterNote+eighthNote, eighthNote, eighthNote, eighthNote, eighthNote, sixteenthNote, sixteenthNote,

quarterNote+eighthNote, sixteenthNote, sixteenthNote, quarterNote, eighthNote, sixteenthNote, sixteenthNote,

quarterNote+eighthNote, sixteenthNote, sixteenthNote, quarterNote, eighthNote, sixteenthNote, sixteenthNote,

quarterNote+eighthNote, sixteenthNote, sixteenthNote, quarterNote, eighthNote, sixteenthNote, sixteenthNote,

quarterNote+eighthNote, sixteenthNote, sixteenthNote, eighthNote, eighthNote, eighthNote, eighthNote

]

# a list of notes for mii

miiChannelNotes = [

NOTE_FS5, NOTE_A5, NOTE_CS6, NOTE_A5, NOTE_FS5,

NOTE_D5, NOTE_D5, NOTE_D5, 0, NOTE_CS5,

NOTE_D5, NOTE_FS5, NOTE_A5, NOTE_CS6, NOTE_A5, NOTE_FS5,

NOTE_E6, NOTE_DS6, NOTE_D6, 0,

NOTE_GS5, 0, NOTE_CS6, NOTE_FS5, 0, NOTE_CS6, 0, NOTE_GS5,

0, NOTE_CS6, 0, NOTE_A5, NOTE_FS5, 0, NOTE_E5, 0,

NOTE_E5, NOTE_E5, NOTE_E5, 0, NOTE_E5, NOTE_E5,

NOTE_E5, 0, NOTE_DS5, NOTE_D5, NOTE_CS5

]

# a list of beat length. Each time has a note that it matches in the list "miiChannelNotes"

miiChannelBeats = [

quarterNote, eighthNote, quarterNote, quarterNote, eighthNote,

eighthNote, eighthNote, eighthNote, halfNote, eighthNote,

eighthNote, eighthNote, eighthNote, quarterNote, quarterNote, eighthNote,

quarterNote, eighthNote, eighthNote, quarterNote+eighthNote,

eighthNote, eighthNote, eighthNote, eighthNote, eighthNote, eighthNote, eighthNote, eighthNote,

eighthNote, eighthNote, eighthNote, eighthNote, eighthNote, eighthNote, eighthNote, eighthNote,

eighthNote, eighthNote, eighthNote, quarterNote+eighthNote, eighthNote, eighthNote,

eighthNote, eighthNote+quarterNote, quarterNote, quarterNote

]

# Display image on the LCD

def displayImg(fileName):

# Loads the bitmap image

bitmap, palette = adafruit_imageload.load(fileName, bitmap=displayio.Bitmap, palette=displayio.Palette)

# Create a TileGrid to store the bitmap

tile_grid = displayio.TileGrid(bitmap, pixel_shader=palette)

# Create a Group to store the TileGrid

group = displayio.Group()

# Add the titleGrid to the Group

group.append(tile_grid)

# Show the group

display.show(group)

def miiProgram():

lowLeft.angle = 90 # set left foot angle

lowRight.angle = 90 # set right foot angle

upLeft.angle = 90 # set left knee angle

upRight.angle = 90 # set right knee angle

# this makes the robot face forward

while 1:

for index in range(0, len(miiChannelBeats), 1):

simpleio.tone(PIEZO_PIN, miiChannelNotes[index], duration=(miiChannelBeats[index])/1.5) # play the miiChannel Song

try:

if (sonar.distance > 25):

return 0 # if the distance of the sonar is greater than 25cm stop playing the song

except RuntimeError:

print("Retrying!")

time.sleep(0.1) # wait 0.1 seconds

# Check if something is within 4cm of sonar

def checkSonar():

try:

distance = sonar.distance # get the distance from the sonar

# check if distance is less than 4cm

if (distance < 4):

miiProgram() # if the sonor distance is less than 4cm run the miiProgram

except RuntimeError:

print("Retrying!") # error message

index = 0   # Initialize global variable to 0

def dance1():

global index # make sure the global varriable index can be accessed

index = 0 # this is the very first dance move. Set index to zero so restart the shooting star song

displayImg("/fireFeet.bmp") # Display corresponding img on LCD

print("Dance 1") # print the word Dance 1

lowLeft.angle = 90 # set angle of left foot

lowRight.angle = 90 # set angle of right foot

upLeft.angle = 90 # set angle of left knee

upRight.angle = 90 # set angle of right foot

for num in range(5):

lowLeft.angle = 60 # set left foot

# Because we only have one processor and we want to do two things at the same time we make it seem like it is threading

# so we play the buzzer

# the buzzer is given a note from the list shootingStarsNotes and its corresponding duration from shootingStarsBeats

# both of these places are determined by the global variable index which is increased by 1 after each note

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

lowLeft.angle = 120 # set left foot angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

lowLeft.angle = 60 # set left foot angle

lowLeft.angle = 90 # set left foot angle

for num in range(5):

upLeft.angle = 60 # set left knee angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

upLeft.angle = 120 # set left knee angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

upLeft.angle = 60 # set left knee angle

for num in range(5):

lowRight.angle = 120 # set right foot angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

lowRight.angle = 60 # set right foot angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

lowRight.angle = 120 # set right foot angle

lowRight.angle = 90 # set right foot angle

for num in range(5):

upRight.angle = 120 # set right knee angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

upRight.angle = 60 # set right knee angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

upRight.angle = 120 # set right knee angle

def dance2():

print("Dance 2")

displayImg("/jumpingJacks.bmp")  #display corresponding image on LCD

global index # make sure the global varriable index can be accessed

if (index >= len(shootingStarsBeats) - 30):

index = 0 # if index is greater than the shootingStar song restart the song

lowLeft.angle = 90 # set left foot angle

lowRight.angle = 90 # set right foot angle

upLeft.angle = 90 # set left knee angle

upRight.angle = 90 # set right knee angle

time.sleep(0.1) # sleep for 1 second

count = 0

while count < 2:

# run this loop twice

upLeft.angle = 130 # set left knee anlge

upRight.angle = 50 # set right knee angle

checkSonar()

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

upLeft.angle = 50 # set left knee angle

upRight.angle = 130 # set right knee angle

checkSonar()

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

lowLeft.angle = 110 # set left foot angle

checkSonar()

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

lowLeft.angle = 90 # set left foot angle

lowRight.angle = 60 # set right foot angle

checkSonar()

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

lowRight.angle = 90 # set right foot angle

lowLeft.angle = 110 # set left foot angle

checkSonar()

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

lowLeft.angle = 90 # set left foot angle

lowRight.angle = 60 # set right foot angle

checkSonar()

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

lowRight.angle = 90 # set right foot angle

lowLeft.angle = 110 # set left foot angle

checkSonar()

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

lowLeft.angle = 90 # set left foot angle

lowRight.angle = 60 # set right foot angle

checkSonar()

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

lowRight.angle = 90 # set right foot angle

count += 1 # increases count by 1

def dance3():

print("Dance 3")

displayImg("/shuffle.bmp")  #display corresponding image on LCD

global index # make sure the global varriable index can be accessed

if (index >= len(shootingStarsBeats) - 30):

index = 0 # if index is greater than the shootingStar song restart the song

lowLeft.angle = 90 # set left foot angle

lowRight.angle = 90 # set right foot angle

upLeft.angle = 90 # set left knee angle

upRight.angle = 90 # set right knee angle

time.sleep(0.1) # sleep for 0.1 seconds

for num in range(60, 120, 20):

if num % 40 == 0:

# if num is a multiple of 40 do the following

lowLeft.angle = num + 10 # set left foot angle

lowRight.angle = num - 10 # set right foot angle

# Because we only have one processor and we want to do two things at the same time we make it seem like it is threading

# so we play the buzzer

# the buzzer is given a note from the list shootingStarsNotes and its corresponding duration from shootingStarsBeats

# both of these places are determined by the global varrialbe index which is increased by 1 after each note

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

lowLeft.angle = num # set left foot angle

lowRight.angle = 180 - num # set right foot angle

upLeft.angle = num # set left knee angle

upRight.angle = 180 - num # set right knee angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

for num in range(120, 60, -20):

upRight.angle = num # set right knee angle

upLeft.angle = 180 - num # set left knee angle

upLeft.angle = num # set left knee angle

upRight.angle = 180 - num # set right knee angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

if num % 40 == 0:

# if num is a multiple of 40 do the following

upRight.angle = num - 10 # set right knee angle

upLeft.angle = num + 10 # set left knee angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

def dance4():

print("Dance 4")

displayImg("/step.bmp") # Display corresponding img on LCD

global index # make sure the global varriable index can be accessed

if (index >= len(shootingStarsBeats) - 30):

index = 0 # if index is greater than the shootingStar song restart the song

for angle in range(70, 110, 20):  # 30 - 150 degrees, 5 degrees at a time.

lowLeft.angle = angle # set left foot angle

# Because we only have one processor and we want to do two things at the same time we make it seem like it is threading

# so we play the buzzer

# the buzzer is given a note from the list shootingStarsNotes and its corresponding duration from shootingStarsBeats

# both of these places are determined by the global varrialbe index which is increased by 1 after each note

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index])  # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

lowRight.angle = 180 - angle # set right foot angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index])  # play the buzzer

index = index + 1 # increase the index by 1

upRight.angle = angle # set right knee angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index])  # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

upLeft.angle = 180 - angle # set left knee angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index])  # play the buzzer

index = index + 1 # increase the index by 1

for angle in range(110, 70, -20):  # 150 - 30 degrees, 5 degrees at a time.

lowLeft.angle = angle # set left foot angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index])  # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

lowRight.angle = 180 - angle # set right foot angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index])  # play the buzzer

index = index + 1 # increase the index by 1

upRight.angle = angle # set right knee angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index])  # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

upLeft.angle = 180 - angle # set left knee angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index])  # play the buzzer

index = index + 1 # increase the index by 1

for angle in range(70, 110, 20):  # 150 - 30 degrees, 5 degrees at a time.

if angle % 90 == 0:

# if the angle is a multiple of 90 do the following

lowLeft.angle = angle - 10 # set left foot angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index])  # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

lowRight.angle = 180 - angle + 10 # set right foot angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index])  # play the buzzer

index = index + 1 # increase the index by 1

upRight.angle = angle - 10 # set right knee angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index])  # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

upLeft.angle = 180 - angle + 10 # set left knee angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index])  # play the buzzer

index = index + 1 # increase the index by 1

lowLeft.angle = angle # set left foot angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index])  # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

lowRight.angle = 180 - angle # set right foot angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index])  # play the buzzer

index = index + 1 # increase the index by 1

upRight.angle = angle # set right knee angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index])  # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

upLeft.angle = 180 - angle # set left knee angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index])  # play the buzzer

index = index + 1 # increase the index by 1

for angle in range(110, 70, -20):  # 150 - 30 degrees, 5 degrees at a time.

if angle % 90 == 0:

# if angle is a multiple of 90 do the following

lowLeft.angle = angle - 20 # set left foot angle

time.sleep(0.1) # sleep for 0.1 seconds

checkSonar()

lowRight.angle = 180 - angle + 20 # set right foot angle

time.sleep(0.1) # sleep for 0.1 seconds

upRight.angle = angle - 20 # set right knee angle

time.sleep(0.1) # sleep for 0.1 seconds

checkSonar()

upLeft.angle = 180 - angle + 20 # set left knee angle

time.sleep(0.1) # sleep for 0.1 seconds

lowLeft.angle = angle # set left foot angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

lowRight.angle = 180 - angle # set right foot angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

upRight.angle = angle # set right knee angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

upLeft.angle = 180 - angle # set left knee angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

def dance5():

print("Dance 5")

displayImg("/dancing.bmp") # Display corresponding img on LCD

global index # make sure the global varriable index can be accessed

if (index >= len(shootingStarsBeats) - 30):

index = 0 # if index is greater than the shootingStar song restart the song

count = 0

# this dance makes it shift backwards

while count < 5:

upRight.angle = 60 # set right knee angle

upLeft.angle = 60 # set left knee angle

checkSonar()

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

upLeft.angle = 120 # set left knee angle

upRight.angle = 120 # set right knee angle

checkSonar()

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

upRight.angle = 60 # set right knee angle

upLeft.angle = 60 # set left knee angle

checkSonar()

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

upRight.angle = 120 # set right knee angle

upLeft.angle = 120 # set left knee angle

checkSonar()

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

upRight.angle = 60 # set right knee angle

upLeft.angle = 60 # set left knee angle

checkSonar()

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

count += 1 # increase count by 1

def dance6():

print("Dance 6")

displayImg("/TipToe.bmp") # Display corresponding img on LCD

global index # make sure the global varriable index can be accessed

if (index >= len(shootingStarsBeats) - 30):

index = 0 # if index is greater than the shootingStar song restart the song

count = 0 # set count to 0

while count < 3:

# run this set of code 3 times

lowLeft.angle = 90 # set left foot angle

lowRight.angle = 90 # set right foot angle

upLeft.angle = 90 # set left knee angle

upRight.angle = 90 # set right knee angle

for angle in range(60, 120, 10):  # 0 - 180 degrees, 5 degrees at a time.

upLeft.angle = angle # set left knee angle

upRight.angle = 180 - angle # set right knee angle

# Because we only have one processor and we want to do two things at the same time we make it seem like it is threading

# so we play the buzzer

# the buzzer is given a note from the list shootingStarsNotes and its corresponding duration from shootingStarsBeats

# both of these places are determined by the global varrialbe index which is increased by 1 after each note

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

if (angle == 90):

lowRight.angle = 120 # set right foot angle

lowLeft.angle = 60  # set left foot angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

lowRight.angle = 60 # set right foot angel

lowLeft.angle = 120 # set left foot angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

lowRight.angle = 120 # set right foot angle

lowLeft.angle = 60 # set left foot angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

for angle in range(60, 120, -10):  # 0 - 180 degrees, 5 degrees at a time.

upLeft.angle = angle # set left knee angle

upRight.angle = angle # set right knee angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

if (angle == 90):

# run this code only if the angle is 90 degrees

lowRight.angle = 120 # set right foot angle

lowLeft.angle = 60 #set left foot angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

lowRight.angle = 60 # set right foot angle

lowLeft.angle = 120 # set left foot angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

lowRight.angle = 120 # set right foot angle

lowLeft.angle = 60 # set left foot angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

upLeft.angle = 90 # set left knee angle

upRight.angle = 90 # set right knee angle

simpleio.tone(PIEZO_PIN, shootingStarsNotes[index], duration=shootingStarsBeats[index]) # play the buzzer

index = index + 1 # increase the index by 1

checkSonar()

count+=1 # increase the count by 1

lowLeft.angle = 90 # set left foot angle

lowRight.angle = 90 # set right foot angle

upLeft.angle = 90 # set left knee angle

upRight.angle = 90 # set right knee angle

time.sleep(1) # sleep for one second

while True:

pass

# initalDisplay()

displayImg("/intro.bmp")

time.sleep(2)

# this is our main function. It runs on a loop and has all our dances

dance1() # dance 1

dance2() # dance 2

dance3() # dance 3

dance4() # dance 4

dance5() # dance 5

dance6() # dance 6

time.sleep(0.5)

displayImg("/splits.bmp")

# do the splits

upLeft.angle = 90 # set left knee angle

upRight.angle = 90 # set right knee angle

lowLeft.angle = 160 # set left foot angle

lowRight.angle = 20 # set right foot angle

time.sleep(5) # wait 5 seconds to restart the dance

Appendix C – Fritzing

Include the snapshot of your fritzing breadboard-view schematic. Include as many as you have, but clearly describe which is which. This is in addition to the fritzing file that you submit to the Canvas.

Below is a photo of the fritzing. It has been submitted on canvas and is also found on github.

Appendix D – GitHub

Every group member must have reasonably and equally contributed to the project github repository. If that is not the case for any member and there is a valid reason as to why, please include an explanation here.

Appendix E – Complete Component list

Include the list of all the components used for the project.

If you have used any component you have purchased on own, include full info, a link to datasheet, and cost.

- One Robot hull

- 2 robot feet

- 4 servo motors

- HC-SR04 Ultrasonic Sensor

- 1 Adafruit ItsyBitsy M4

- 1 Adafruit 1.44 inch TFT Breakout

- 1 Piezo Speaker

- A whole lot of wire

Appendix F – Answer the following questions:

Q1 – Teamwork: Explain in details the methods your group has used to communicate effectively among team members.

To communicate with one another, we created a Facebook messenger group chat. This allowed us to communicate effectively and quickly, especially during our reading break. In addition, we developed a Gantt chart to distribute the work and ensure we remained on schedule. This chart kept track of tasks that needed to be done, who oversaw the task and when it was due.

Q2 – Design Process for the additional functionalities: Describe clearly the process you used for the following design aspects of your own additional functialities. Please spend time to carefully answer each of them.

- Use of process: Describe your approach to adapt and apply a general design process for any additional feature. What was your approach?

When conducting this project, we followed the Gantt chart and divided the work equally. For more difficult aspects of the project, two people or all of us would meet to work together to develop solutions and build what was necessary. This included building the robot, programing the LCD screen and working on the buzzer or the ultrasonic sensor. After doing our individual parts, we would review the one another’s code. This helped us quickly fix bugs, get a new perspective on our individual task and integrate these components quickly into the final product.

- Constraint identification: Explain the constraints that you must consider in the design of the additional functionalities.

  - Constraint 1: The biggest constraint we had was the amount of ports on the ItsyBitsy. To fix this constraint, we could have used a shift register.

  - Constraint 2: When introducing additional features, we found that it would sometimes slow down the main functionality of our program. For example, when syncing the moves with the music, we attempted to introduce our own version of threading by continuously switching between a movement and then a buzzer sound. We found that when we executed this, it made the dance moves look very rigid and broken. We had to find a way to sync the music and moves while keeping the flow and movement appealing.

- Solution generation: Explain at least two possible alternative additional features that your group rejected due to technical reasons and explain why.

  - Because of the amount of ports on the ItsyBitsy, we decided to not go with the keypad. We could have integrated this into the robot with a shift register, however, but we decided that the benefit when compared to the amount of work required was not worth it.

  - We also decided to not go with the LED light show. The ultrasonic sensor and buzzer would be better additions in place of the LED light show because they had more of an impact on the design and outcome of the robot, and allowed us more flexibility in our design.

- Solution Assessment: Explain how you tested and assessed the viability and then correctness of your group’s additional features.

  - To be successful in this project, testing, assessing viability and overall correctness was vital. Before integrating any part into the project, we tested the code and its implication extensively. This is shown in our GitHub repository. Each dance has its own file, and each one was tested rigorously. This reduced errors in the overall project and code and allowed our results to be more correct. This ideology was applied in all aspects of creating the robot.

Appendix G - Other

Include any other relevant info that does not fit in any other section in the report.